# Purpose

Gel electrophoresis is a common laboratory technique used to separate DNA fragments by size. This process involves moving negatively charged DNA through an agarose gel matrix using an electric field. Shorter DNA fragments travel faster than longer ones. By comparing the DNA fragments to a DNA ladder (a set of known lengths), you can determine the success of DNA extractions and PCR, and you can estimate the fragment length distribution of your DNA samples.

# Scope

Test gels are most appropriate to do prior to sequencing to ensure PCR has been successful and to save costs of needless sequencing of unamplified PCR templates.

# Responsibilities

This protocol should be executed by and/or under the supervision of trained personnel.

# Definitions

- Agarose: A polysaccharide extracted from seaweed, used to create a gel matrix for electrophoresis. It forms a porous gel that allows the separation of DNA fragments based on size.

- TAE Buffer (Tris-Acetate-EDTA): A buffer solution used in gel electrophoresis to maintain a stable pH and conduct electricity during the separation of DNA.

- Ethidium Bromide (EtBr): A fluorescent dye that intercalates with DNA and emits light when exposed to UV light, allowing visualization of DNA bands in the gel.

- Gel Electrophoresis: A technique used to separate DNA fragments by size. DNA is negatively charged and moves through an agarose gel towards a positive electrode when an electric field is applied.

- DNA Ladder: A set of known DNA fragments with defined sizes used as a molecular weight marker to estimate the size of DNA fragments in the sample.

- Loading Buffer: A solution added to DNA samples before loading them into the gel. It contains a dye for tracking and glycerol to increase sample density, ensuring the samples sink into the wells.

- Gel Box (Electrophoresis Unit): The apparatus used to hold the agarose gel and buffer during electrophoresis. It includes electrodes connected to a power source to generate the electric field.

- Microwave Pulsing: The technique of heating the agarose solution in short bursts in a microwave, with intermittent stirring to prevent boiling over and ensure complete dissolution.

- UV Light Source: A device that emits ultraviolet light used to visualize DNA fragments stained with ethidium bromide in the gel.

- Well Comb: A comb-like tool placed in the gel casting tray before the agarose solidifies to create wells where DNA samples will be loaded.

- Casting Tray: A tray used to shape and solidify the agarose gel. It holds the well comb in place and provides the form for the gel.

- Voltage Source: The power supply used to apply an electric field across the gel, causing the DNA fragments to migrate.

- Mutagen: A chemical agent that can cause changes (mutations) in the DNA sequence. Ethidium bromide is an example of a mutagen used in this SOP.

- Electrodes: Conductive elements in the gel box that create the electric field when connected to the voltage source, directing the movement of DNA through the gel.

- Running Buffer: The buffer solution (usually TAE or TBE) that fills the gel box and conducts the electric current during electrophoresis.

# References

Addgene. (2018, February 20). Protocol - How to Run an Agarose Gel. Retrieved from Addgene website.

# Materials

| Material Description | Standard Quantity | Unit |
| --- | --- | --- |
| Casting tray | N/A | N/A |
| Well combs | N/A | N/A |
| Voltage source | N/A | N/A |
| Gel box | N/A | N/A |
| UV light source | N/A | N/A |
| TAE buffer (see recipe) | N/A | N/A |
| Agarose | N/A | N/A |
| Ethidium bromide (EtBr) (stock concentration: 10 mg/mL) | N/A | N/A |

# Safety

- Always wear a lab coat, gloves, and safety goggles to protect against chemical exposure.

- Ethidium bromide (EtBr) is a mutagen. Handle it with care, wear PPE, and use it in a designated area with proper waste disposal methods. Avoid skin contact and inhalation.

- Use microwave-safe flasks and be cautious of eruptive boiling when heating the agarose solution. Microwave in short pulses and swirl the solution intermittently to prevent overheating.

- The agarose solution will be very hot after microwaving. Use heat-resistant gloves when handling hot flasks and allow the solution to cool to a manageable temperature before pouring.

- Ensure that the gel box and power supply connections are secure and dry before starting electrophoresis. Avoid touching the buffer or gel while the apparatus is connected to the power supply.

- When visualizing DNA bands under UV light, wear UV-protective face shields or goggles, and cover exposed skin to avoid UV radiation exposure.

- Dispose of used gels, gloves, and other materials contaminated with EtBr in designated biohazard containers. Follow your institution’s guidelines for hazardous waste disposal.

- Familiarize yourself with the location of emergency equipment such as eyewash stations, safety showers, and first aid kits. Know the emergency procedures for your lab.

# Procedure

Pouring a Standard 1% Agarose Gel

Measure Agarose

- Weigh 1 gram of agarose.

- Tip: Agarose concentration can vary from 0.7% to 2% based on the band size you need to separate. Adjust the agarose mass accordingly.

Mix Agarose with TAE Buffer

- Mix 1 g of agarose with 100 mL of 1x TAE in a microwavable flask.

- Tip: TBE can also be used, but stick to one type of buffer.

Dissolve Agarose

- Microwave the mixture for 1-3 minutes until completely dissolved. Do not overboil.

- Caution: The solution will be hot and can cause eruptive boiling. Microwave in short pulses, stirring occasionally.

Cool the Solution

- Allow the agarose solution to cool to about 50°C (about 5 minutes).

Add Ethidium Bromide

- Add EtBr to a final concentration of 0.2-0.5 μg/mL (2-3 μL of stock solution per 100 mL gel).

- Caution: EtBr is a mutagen. Wear protective gear.

Pour the Gel

- Pour the agarose into a casting tray with a well comb in place.

- Tip: Pour slowly to avoid bubbles. Push any bubbles away from the wells using a pipette tip.

Solidify the Gel

- Let the gel solidify at 4°C for 10-15 minutes or at room temperature for 20-30 minutes.

Procedure

Loading Samples and Running the Gel

Add Loading Buffer

- Mix loading buffer with each sample of your DNA extraction or PCR product (e.g. 1 µL of 6x loading buffer with 5 µL of DNA sample or 2 µL of 2x loading buffer with 2 µL of DNA sample).

- Note: Loading buffer contains dye for visualization and glycerol to make the sample dense.

Place the Gel in the Gel Box

- Once solidified, place the gel in the gel box and cover with 1x TAE buffer.

- Tip: If you added EtBr to the gel, also add it to the buffer.

Load the DNA Ladder

- Carefully load a molecular weight ladder into the first lane.

Load DNA Samples

- Load your DNA samples into the additional wells.

Run the Gel

- Run the gel at 80-150 V until the dye is about 75-80% down the gel (typically 1-1.5 hours).

- Follow manufacturer instructions of gel rig to power on a adjust voltage settings.

- Note: Ensure the DNA runs toward the positive electrode (Run to Red).

Turn Off Power and Remove Gel

- Turn off the power and carefully remove the gel.

Visualize DNA

- Use a UV light source to visualize DNA bands following manufacturer instructions for your UV visualization platform.

Analyzing Gel

- Compare the DNA bands to the DNA ladder to estimate their sizes.

# Figures and Diagrams

N/A

# Notes/Observations

- Resolution of Bands: Run the gel at a lower voltage, use a wider/thinner comb, or load less DNA.

- Separation of Bands: Adjust the agarose percentage for better separation.

- 10% Rule: Make 10% more volume of each sample to account for pipetting loss.

# Attachments

N/A