# Purpose

For purification of DNA from plant leaf tissue.

# Scope

This protocol is suited for DNA extraction from dry leaf samples of non-succulent and non-evergreen plant species.

# Responsibilities

Trained students should be supervised by qualified personnel.

# Safety

# PPE (including lab jacket, gloves, and safety glasses) must be worn when following SOP and in the lab vicinity. Change gloves or disinfect gloves with isopropanol or ethanol between steps to avoid contamination. Wash hands and disinfect surfaces with ethanol or isopropanol before and after performing SOP. Do not dispose of lysis buffer or wash I buffer down the sink (ask instructor/supervisor of special waste containers).

# Definitions

- DNA (Deoxyribonucleic Acid): The hereditary material in almost all organisms, containing the instructions needed for an organism to develop, survive, and reproduce.

- Plant Leaf Tissue: The source material from plants used for DNA extraction in this protocol.

- MagMAX™ Plant DNA Isolation Kit: A kit containing reagents and materials necessary for extracting DNA from plant tissues.

- Heat Block/Thermomixer: Equipment used to maintain a constant temperature for heating samples during the extraction process.

- Centrifuge (≥16000 × g): A device used to separate components of a sample based on density by spinning them at high speed.

- Centrifuge with Swinging-Bucket Rotor: A centrifuge designed to hold plates and spin them at lower speeds (3000–4000 × g).

- Micropipettors (Adjustable/Multichannel): Instruments used to accurately measure and transfer small volumes of liquid.

- Vortex Mixer: A device used to mix samples quickly by creating a vortex in the liquid.

- Nonstick RNase-Free Microfuge Tubes (1.5 mL): Tubes designed to prevent RNA degradation and used for various sample preparation steps.

- MicroAmp™ Clear Adhesive Film: A film used to seal plates during the purification process to prevent contamination and evaporation.

- Aerosol-Resistant Pipette Tips: Tips designed to prevent contamination by aerosols during pipetting.

- Reagent Reservoirs: Containers used to hold reagents temporarily during pipetting.

- Disposable Gloves: Personal protective equipment (PPE) to protect the user and prevent contamination.

- Ethanol (96–100%): A molecular biology grade alcohol used for washing and purifying DNA.

- Pestle: A tool used for grinding plant tissue in microfuge tubes.

- KingFisher™ Flex Magnetic Particle Processor: An automated instrument used for purifying nucleic acids using magnetic particles.

- KingFisher™ 96 Deep-Well Plate: A plate designed for use with the KingFisher Flex for high-throughput processing.

- KingFisher™ 96 KF Microplate (200 µL): A microplate used with the KingFisher Flex for smaller volume processing.

- 96 Deep-Well Tip Combs: Combs used in conjunction with the KingFisher Flex for handling samples during magnetic particle processing.

- KingFisher™ Elution Strip: A strip used to collect purified DNA after processing with the KingFisher Duo Prime Magnetic Particle Processor.

- Magnetic Stand: A stand used to hold tubes and plates during manual magnetic separation steps.

- Ice Bucket/Chill Rack: Equipment used to keep samples cold during preparation steps to prevent degradation.

# References

MagMAX Plant DNA Isolation Kit User Guide

# Materials

(Any materials here)

| Material Description | Standard Quantity | Unit |
| --- | --- | --- |
| MagMAX™ Plant DNA Isolation Kit (Cat. Nos. A32549, A32580, and A47156) | 1 | N/A |
| Heat block, water bath, or thermomixer at 65°C | 1 | N/A |
| Centrifuge capable of ≥16,000 × g for microcentrifuge tubes | 1 | N/A |
| Centrifuge capable of 3000–4000 × g with swinging-bucket rotor for 96-well deep well plates | 1 | N/A |
| Adjustable micropipettors | N/A | N/A |
| Multi-channel micropipettors | N/A | N/A |
| Vortex mixer, or equivalent | 1 | Kit |
| Nonstick, RNase-free 1.5 mL microfuge tubes (AM12450) | N/A | N/A |
| MicroAmp™ Clear Adhesive Film (4306311) for purification in plate format | N/A | N/A |
| Aerosol-resistant pipette tips | N/A | N/A |
| Reagent reservoirs | N/A | N/A |
| Disposable gloves | N/A | N/A |
| Ethanol, 96–100% (molecular biology grade) | N/A | N/A |
| Pestle for 1.5 mL microfuge tubes | N/A | N/A |
| KingFisher™ Flex Magnetic Particle Processor with 96 Deep-Well Head (5400630) | 1 | N/A |
| KingFisher™ 96 Deep-Well Plate (95040450) | N/A | N/A |
| KingFisher™ 96 KF microplate (200 µL) (97002540) | N/A | N/A |
| 96 Deep-Well Tip Combs for KingFisher™ Flex Magnetic Particle Processor (97002534) | N/A | N/A |
| (For KingFisher™ Duo Prime Magnetic Particle Processor) KingFisher™ Elution Strip (97003520) | N/A | N/A |
| Magnetic stand, compatible with selected manual purification format (AM10027) | N/A | N/A |
| Ice bucket or chill rack | N/A | N/A |

# Procedure

Before beginning

- Make sure the following program is loaded in the KingFisher Flex using graphic user interface: “A32549_Plant_DNA_Flex 96DW”. Refer to manufacturer’s manual for program installation if need.

- For first-time use of wash solution 1, check the box on the bottle label to indicate that this step has been completed. If not, add 135 mL of 96–100% ethanol to Wash Solution 1 Concentrate, initial the box and timestamp the date of mixing.

- For first-time use of wash solution 2, check the box on the bottle label to indicate that this step has been completed. If not, add 180 mL of 96–100% ethanol to Wash Solution 2 Concentrate, mix, initial the box and timestamp the date of mixing.

- Check all the solutions in the kit for salt precipitation. Dissolve any precipitates by warming the solution at 37°C, then equilibrate to room temperature.

- Resuspend the DNA Binding Beads well by vortexing.

- Set heat block, water bath, or thermomixer to 65°C.

- Get ice bucket or pre-chill an ice rack.

- Ensure that the instrument is set up for processing with the deep-well magnetic head and has the standard 96-well heating block installed.

Manual tissue disruption

- Add lysis buffer A (350 µL) to each 1.5 mL tube labelled with sample ID (instructor may have already done for you).

- For broadleaf tissue, fold sampled leaf in half and punch a hole in leaf in each 1.5 mL microfuge tube using the cap. This is approximately 10 – 100 mg for fresh leaf tissue. For dry leaf tissue measure and use no more than 20 mg.

- Using a clean pestle for microfuge tubes, grind a sample in its tube for five minutes at room temperature.

- Vortex 15 seconds and briefly spin down (1-2 seconds before reaching set speed) with tubes opposite each other.

- Add lysis buffer B (50 µL) and RNase (50 µL).

- Vortex to mix, incubate for 10 minutes in the 65°C heat block, water bath, or thermomixer while vortexing intermittently.

- After incubating, add precipitation buffer (130 µL).

- Flip closed tubes upside down 2-3 times to mix.

- Let tubes sit on ice or chill rack for 5 minutes.

- Place pairs of tubes of equal volume opposite each other in centrifuge and spin down for five minutes at max speed (≥ 16,000 × g).

- Transfer 400 µL of the clear lysate (supernatant) to a pre-assigned (and documented in lab notebook or attachment) well of a clean 96-well plate or a clean 1.5 mL tube labelled with sample ID, “lysate”, and date of lysis (instructor may have done this for you).

- Transer sample plate/tube with date and accession/sample information recorded in attachment to batch record. Store clarified lysates in the freezer for use within seven days or proceed with automated DNA purification.

Automated purification with KingFisher Flex

- Setup plate of Wash 1 solution by dumping a sufficient amount of reagent in clean reservoir tray, pipetting with a multichannel 800 µL of reagent into each well of a deepwell plate being used, and placing plate into position 2 of KingFisher Flex. Be sure A1 well of plate is oriented according to markings in position 2 slot.

- Setup plate of Wash 2 solution by dumping reagent in clean reservoir tray, pipetting with a multichannel 800 µL of reagent into each well being used, and placing plate into position 3 of KingFisher Flex.

- Setup another plate of Wash 2 solution by dumping reagent in clean reservoir tray, pipetting with a multichannel 800 µL of reagent into each well being used, and placing plate into position 4 of KingFisher Flex.

- Setup plate of Elution solution by dumping reagent in clean reservoir tray, pipetting with a multichannel 150 µL of reagent into each well being used, and placing plate into position 5 of KingFisher Flex.

- Place tip comb on standard plate in position 6 of KingFisher Flex.

- For each well of sample plate being occupied, add 25 µL DNA Binding Beads that have been recently vortexed before poured in reservoir tray. To avoid contamination, change tips of multichannel/micro-pipette if any come in contact with prior sample.

- For each well of sample plate being occupied, add 400 µL (with multichannel or micro-pipette) of freshly mixed 96-100% ethanol that has been poured into reservoir tray.

- Place sample plate in position one of KingFisher Flex Instrument.

Run Bind, wash, and elution program on KingFisher Flex

- Ensure the instrument is setup with both the deep-well and standard 96-well heating block installed. Make sure all plates are oriented correctly (see A1 labels around positions where plates are inserted).

- Turn on KingFisher Flex with switch on the left side of machine.

- Use arrow buttons on panel to navigate to program “A32549_Plant_DNA_Flex 96DW” program (go to middle tab on panel and scroll down to DNA, click ‘OK’, go to “A32549_Plant_DNA_Flex96DW”, and click ‘START’).

- Follow prompts of instrument panel to confirm proper placement of each reagent plate, tip comb, and sample plates, hitting ‘START’ between each prompt and once more to start program.

- At the end of the run (approximately 39 minutes), remove elution plate and transfer eluate to new tubes (noting sample identity directly on tubes) or seal plate and record information matching well number of sample plate with the identity record of sample.

- Note tube/plate ID and location (freezer ID, box ID) of tubes/plates in batch record/notebook and later in electronic records.

# Figures and Diagrams

N/A

# Notes/Observations

# Attachments

Accession-Sampling Sheet Template

VG004 Attachment - MagMax-KingFisher DNA Extraction Batch Record